Development Notes

Topics covered in this document include:

- Routers

- Router configurations

- Headers

Router Configuration

Router configurations are written into .ini files. Each file is of the following format:

Section 1: metadata

HOST = real ip address of the device running the router program.

PORT = real port number for the router’s socket. My convention is to make the port 60000 + virtual_address.

Section 2: host

virtual_address = integer address number.

bandwidth = Number in Mbps

buffer_size = Number in megabytes.

Section 3: routing

neighbors = multiline list of virtual addresses which are considered to be “neighboring” this router in the network topology.

destinations = multiline list of virtual addresses which belong to hosts.

local_forwarding = multiline list consisting of neighboring virtual addresses. The index of each entry matches the corresponding destination which that address is pointing towards.

Header Formatting

Headers are 8 bytes long and have the following format:

1 byte: TTL

1 byte: Message Type Flags

1 byte: Message identifier

1 byte: Reserve space (for future additions

2 bytes: Source virtual address

2 bytes: Destination virtual address.

The payload of the packet follows the header.

Datagram Implementation

Datagram headers are structured as python structs, with the format "!BBBHHH". They are big-endian, as is indicated by the’!’ at the beginning of the format string.

Datagram payloads are utf-8 encoded bytestrings.

Router Datagram Handling

A packet is dropped if its TTL is less than or equal to zero.

A packet addressed to a router will be logged by the router and then discarded. This behavior can be changed if needed.

A router spends most of its time listening to its socket. After receiving a packet, it will forward or discard it as needed.

Known Limitations

Router forwarding tables are currently static. Routers cannot generate tables of their own.

Routers currently assume that the network is being hosted on one device. Each router attempts to send packets to the local host IP. Routers do not have a way of knowing the IPs of neighbors beyond this assumption.

Routers assume that other routers use the port convention of 60,000+virtual address. As such, our system’s scalability is limited.